AI BLASTING SUITE

# User Manual

Operating instructions, module purposes, inputs, outputs, formats and units

| Document field | Value |
| --- | --- |
| Software | AI Blasting Suite — React/FastAPI web application |
| Manual version | 1.0 — repository baseline |
| Prepared | 13 August 2026 |
| Intended users | Blasting engineers, reviewers, planners, analysts, trainers and authorised students |
| Status | Draft for owner review and site-specific completion |

SAFETY-CRITICAL SCOPE This software supports analysis, planning, education and simulation. It does not arm, fire, program or communicate with detonator hardware. Every design, prediction and export must be checked and approved by qualified blasting personnel under applicable legislation, mine procedures, manufacturer requirements and a site-specific risk assessment.

OWNER REVIEW REQUIRED Yellow boxes identify information or screenshots that could not be authoritatively confirmed from the application and repository. The software owner should complete these items before releasing this manual as a controlled document.

# Document control and approval

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Document owner | [OWNER TO COMPLETE] |  |  |
| Technical reviewer | [OWNER TO COMPLETE] |  |  |
| Qualified blasting reviewer | [OWNER TO COMPLETE] |  |  |
| Release approver | [OWNER TO COMPLETE] |  |  |

## Revision history

| Version | Date | Author/owner | Change |
| --- | --- | --- | --- |
| 1.0 | 2026-08-13 | Generated from the current application baseline | Initial comprehensive user manual |

OWNER ACTION REQUIRED — DOCUMENT METADATA Add the production URL, organisation name/logo, document identifier, software release/build number, support contact, formal approval names and next review date.

# Contents

| 1. About this manual | 9. Delay Design & Simulation |
| --- | --- |
| 2. Access, sign-in and sign-out | 10. Slope Stability |
| 3. Workspace orientation | 11. Back Break |
| 4. Data Manager | 12. Flyrock (ML + Empirical) |
| 5. Prediction | 13. File formats and output handling |
| 6. Feature Importance & Explainable AI | 14. Troubleshooting |
| 7. Parameter Optimisation | 15. Glossary |
| 8. Cost Optimisation | 16. Owner completion checklist |

# 1. About this manual

## 1.1 Purpose

This manual explains how authorised users sign in, prepare data, run each analysis or planning module, interpret its outputs and export results. It covers the web application currently represented by the repository baseline.

## 1.2 What the suite does

AI Blasting Suite combines dataset quality checks, empirical calculations, machine-learning predictions, explainability, design optimisation and blast-timing simulation in one workspace. Its modules share the active combined dataset where indicated.

## 1.3 Intended audience

Blasting engineers and licensed blasting personnel performing qualified review.

Technical services, planning and operations personnel comparing design scenarios.

Geotechnical and safety personnel screening slope, flyrock and back-break risk.

Data analysts maintaining datasets and reviewing model behaviour.

Students and trainers using the planning and simulation functions under supervision.

## 1.4 Safety and decision limitations

Predictions and optimised recipes are decision-support outputs, not approvals or firing instructions.

Machine-learning quality depends on the relevance, size and quality of the training dataset.

Empirical constants and acceptance limits must be calibrated for the site.

The Delay Design & Simulation module is a planning draft tool and cannot control explosives or initiation hardware.

A low predicted risk or a model classification never replaces engineering judgement, inspections, statutory controls or exclusion-zone procedures.

## 1.5 Conventions and common units

Screenshots in this manual were captured in a local demonstration environment with sample data and authentication disabled. Production URLs, user identity, available datasets and calculated values can differ; the controls and workflow remain representative of the documented baseline.

| Symbol/term | Meaning | Unit or format |
| --- | --- | --- |
| B / Burden | Distance from a blasthole row to the free face or preceding row | m |
| S / Spacing | Distance between holes in a row | m |
| HPD | Holes firing in the same delay group | count (dimensionless) |
| PPV / Ground vibration | Peak particle velocity | mm/s |
| Airblast | Air overpressure level | dB |
| X50 | Fragment size at 50% passing | mm |
| Xm | Kuz–Ram mean fragmentation size | mm |
| PF | Powder factor | kg/m³ |
| RWS | Relative weight strength | dimensionless relative value |
| R² | Model coefficient of determination | dimensionless; higher is generally better |
| Probability | Model likelihood or screening score | 0–1 internally; often displayed as % |
| Delay | Time from initiation reference | ms |
| Coordinates | X/easting, Y/northing and optional Z/elevation | Project coordinate units; normally m, confirm site system |
| Cost | Initiation, explosive, drilling and total cost | BWP in the current interface |

# 2. Access, sign-in and sign-out

## 2.1 Current configured access list

The following addresses are the default allowlist found in both the frontend and backend configuration:

Authorised email

so13000604@biust.ac.bw

Ozigwa@debswana.bw

Tgalefete@debswana.bw

Mhiya@debswana.bw

Ttshambane@debswana.bw

Mgaopelo@debswana.bw

CONFIGURATION NOTE The deployed environment can override this default list through VITE_ALLOWED_LOGIN_EMAILS and BLAST_ALLOWED_AUTH_EMAILS. Treat the table above as the repository baseline, not proof of the live production allowlist.

The current application implements an email allowlist rather than separate user roles. An authorised user can navigate to the same user-facing modules; organisational approvals and operating authority remain outside the software.

OWNER ACTION REQUIRED — VERIFY ACCESS Before issue, compare this table with the current production environment allowlist and the organisation's access approval record. Add an access owner and review frequency. Do not add passwords or verification codes.

## 2.2 Sign in with a one-time code

1. Open the approved production URL in a current browser.

2. Enter an authorised work email address.

3. Select Send magic code.

4. Retrieve the one-time code from the mailbox and enter it in Verification code.

5. Select Verify and continue. If the email or code is rejected, see Troubleshooting.

Figure 1 — Passwordless sign-in screen.

## 2.3 Session and sign-out

The browser stores an InstantDB refresh token and the signed-in email in local storage so a session can be restored after refresh. Use Sign out in the application header when finished, especially on a shared workstation. Do not copy browser storage values or authentication tokens into support requests.

# 3. Workspace orientation

## 3.1 Navigation groups

| Navigation group | Modules |
| --- | --- |
| Analysis | Prediction; Feature Importance; Parameter Optimisation |
| Operations | Cost Optimisation; Delay Design & Simulation |
| Safety / Geo | Slope Stability; Back Break; Flyrock |
| Admin | Data Manager |

## 3.2 Header controls

Dataset selects the shared combined dataset used by Data Manager defaults, Prediction, Feature Importance and Parameter Optimisation.

Theme selects System, Light or Dark appearance.

Accent selects blue, green or dark-blue interface accents.

Backend status indicates whether the service responds. Analysis requiring the backend will fail when it is unavailable.

The user email and Sign out control identify and end the current session.

## 3.3 Welcome dashboard

Current user, last app use and last module used summarise recent activity.

Module guide cards explain each module and provide Open module shortcuts.

Quick access opens Prediction, Cost Optimisation or Flyrock.

Export professional report creates printable HTML containing the active dataset, module guide, generating user and recent user/module/timestamp activity. Review this personal and operational metadata before distributing the report.

Recent activity is a convenience summary, not a complete security or statutory audit log.

DATASET CHANGE EFFECT Changing the combined dataset resets prediction and optimisation model caches. The next request may take longer while models are prepared again.

Figure 2 — Welcome dashboard and main navigation.

## 3.4 Recommended end-to-end workflow

1. Confirm that the correct combined dataset is selected.

2. Use Data Manager to inspect ranges, missing values, correlations and compliance filters.

3. Run Prediction for the proposed design and compare empirical and ML results.

4. Use Feature Importance to understand influential variables and data structure.

5. Use Parameter Optimisation or Cost Optimisation to compare scenarios and trade-offs.

6. Use Slope Stability, Back Break and Flyrock for additional safety screening.

7. Use Delay Design & Simulation to create and review a planning draft timing sequence.

8. Export results, record dataset/version details and obtain qualified review.

# 4. Data Manager

| Purpose | Use this module when | Primary outputs |
| --- | --- | --- |
| Load, inspect, append, filter, visualise, calibrate and export tabular site datasets. | Preparing or quality-checking data before prediction, explainability or optimisation. | Filtered CSV, nominal Excel export, site-model JSON, statistics, audits and plots. |

## 4.1 Input format

| Requirement | Details |
| --- | --- |
| Accepted file | Comma-separated values (.csv) |
| Header | First non-empty row; unique descriptive column names recommended |
| Values | One observation per row; numeric analysis columns should contain decimal numbers without unit text |
| Append behaviour | Creates the union of both files' columns and appends rows; missing fields remain blank |
| Shared effect | The loaded data becomes the active in-browser dataset for Prediction, Feature Importance and Parameter Optimisation |

## 4.2 Controls and outputs

| Tab/control | How to use it | Output |
| --- | --- | --- |
| Load CSV | Replace the current in-browser data with a CSV. | Table and row/column count |
| Append CSV | Add rows from another CSV; review column alignment. | Merged table |
| Table | Search text across all fields and add rows. | Filtered display or added row |
| Summary | Review descriptive statistics and enter PPV limit, air limit and HPD. | Statistics and quick compliance audits |
| Visuals | Choose plot type and X/Y columns; optional logarithmic axes. | Scatter, line, bar, histogram, box, hexbin or engineering plot |
| Correlations | Review numeric-column relationships. | Correlation heatmap |
| Filters | Enter numeric minimum/maximum values or a row query. | Filtered rows |
| Calibration | Enter HPD and run PPV, airblast or fragmentation calibration. | Calibration log and site_model.json |
| Export | Export the currently filtered rows. | filtered.csv or nominal filtered.xlsx |

CURRENT FORMAT LIMITATION The control labelled “Export Filtered → Excel” currently writes comma-separated text but names the file filtered.xlsx. Use the CSV export for a reliable file, or open the exported content as CSV and save it as a true Excel workbook. The software owner should correct or validate this behaviour before relying on the Excel label.

## 4.3 Procedure

1. Select Load CSV and choose the approved dataset.

2. Confirm the displayed row and column counts.

3. Review Table and Summary for blanks, implausible values and unit consistency.

4. Use Visuals and Correlations to identify outliers and strongly related columns.

5. Apply Filters or a query to create the required subset.

6. If calibration is required, enter HPD and run only the calibration supported by suitable measured outputs.

7. Export the filtered CSV and record the source file, filter criteria and date.

QUERY EXAMPLE Use backticks around column names containing spaces, for example: `Ground Vibration` <= 12.5 and `Airblast` <= 134. Queries use comparison expressions; use only approved expressions and never paste untrusted code into this field.

Figure 3 — Data Manager table and tabs.

Figure 3A — Data Manager descriptive statistics and quick audits.

Figure 3B — Data Manager correlation heatmap.

# 5. Prediction

| Purpose | Use this module when | Primary outputs |
| --- | --- | --- |
| Estimate ground vibration, airblast and fragmentation for one blast design using empirical equations and optional machine learning. | Comparing a proposed design with site limits and checking empirical/ML agreement. | Output comparison chart, Rosin–Rammler curve, alerts, prediction_result.csv and printable HTML report. |

## 5.1 Blast inputs

| Input | Purpose | Unit |
| --- | --- | --- |
| Hole depth | Drilled hole length used for charge and geometry calculations. | m |
| Hole diameter | Nominal blasthole diameter. | mm |
| Burden | Distance to free face/preceding row. | m |
| Spacing | Hole spacing within a row. | m |
| Stemming | Uncharged collar length. | m |
| Distance | Distance from blast to monitoring/receptor point. | m |
| Powder factor | Explosive mass per blast volume. | kg/m³ |
| Rock density | Bulk rock density. | t/m³ |
| Linear charge | Explosive mass per charged metre. | kg/m |
| Explosive mass | Explosive mass represented by the row/design. | kg |
| Blast volume | Rock volume represented by the design. | m³ |
| # Holes | Number of blastholes. | count |

## 5.2 Empirical and RR settings

| Input | Default | Purpose |
| --- | --- | --- |
| K_ppv | 1000 | Site vibration constant in the scaled-distance PPV equation |
| β | 1.60 | Vibration decay exponent |
| K_air | 170 | Airblast intercept/calibration constant |
| B_air | 20 | Airblast logarithmic slope |
| A_kuz | 22 | Kuz–Ram rock factor |
| RWS | 115 | Relative explosive strength |
| HPD | 1 | Holes contributing to charge per delay |
| Oversize threshold | 500 mm | Size above which RR oversize percentage is calculated |
| RR n mode | Estimate | Estimate uniformity n or use a manual value |
| Manual n | 1.8 | Rosin–Rammler uniformity parameter when Manual is selected |
| Use ML | Yes | Include available ML predictions; No uses empirical outputs only |

## 5.3 Outputs and interpretation

| Output | Unit | Interpretation |
| --- | --- | --- |
| Ground Vibration / PPV | mm/s | Compare with the approved site/receptor limit. |
| Airblast | dB | Compare with the approved site/receptor limit. |
| Fragmentation | mm | Predicted characteristic size; confirm which statistic the selected dataset uses. |
| RR n | dimensionless | Higher values indicate a narrower particle-size distribution. |
| Xm / X20 / X50 / X80 | mm | Characteristic fragmentation sizes from the RR curve. |
| Oversize | % | Predicted fraction above the selected oversize threshold. |
| Threshold log | text | Flags outputs that exceed user-entered thresholds; checks ML first when available. |

## 5.4 Procedure

1. Confirm the dataset source shown at the top of the module.

2. Enter each blast input, or select Use Medians to initialise values from the current data.

3. Enter site-approved empirical constants, HPD and RR settings.

4. Enter threshold values for any required alerts.

5. Select Predict (ML + Empirical).

6. Compare ML and empirical outputs; investigate material disagreement instead of choosing the preferred answer.

7. Open Fragmentation (RR Curve) and review X20/X50/X80 and oversize.

8. Export the CSV or professional report and retain the dataset and constants with the result.

Figure 4 — Prediction inputs and empirical/ML output comparison.

Figure 4A — Prediction Rosin–Rammler fragmentation curve.

# 6. Feature Importance & Explainable AI

| Purpose | Use this module when | Primary outputs |
| --- | --- | --- |
| Explain which inputs drive model outputs and reveal correlation or low-dimensional structure in the active dataset. | Prioritising design variables, checking redundant inputs or reviewing model behaviour after prediction. | RF importance, permutation sensitivity, local impact waterfall, partial dependence, correlation map and PCA plots. |

## 6.1 Inputs

Active shared or uploaded dataset from Data Manager.

Top-K features: integer from 5 to 30; default 12.

Explainability output: select the model output to inspect after computation.

## 6.2 Procedure and interpretation

1. Confirm the dataset name and source shown in the module.

2. Choose Top-K and select Compute RF Importance.

3. Check mapping mode, rows used and rows dropped. Positional split means the application could not map standard names and assumed the final three columns are outputs.

4. Compare global RF importance with permutation sensitivity. A variable that is strong in both views is generally more credible than one strong in only one metric.

5. Select an output and review local impacts and partial dependence for direction, not causation.

6. Run PCA Analysis to review explained variance, PC1/PC2 separation and loading variables.

7. Use the findings to choose optimisation axes, but retain engineering constraints and site knowledge.

INTERPRETATION CAUTION Importance and correlation do not prove causation. Correlated features can share or mask importance, and extrapolation outside the dataset range is unreliable.

Figure 5 — Feature importance and explainability results.

# 7. Parameter Optimisation

| Purpose | Use this module when | Primary outputs |
| --- | --- | --- |
| Build a physics-informed surrogate from the active dataset, search Pareto candidate recipes and perform inverse design. | Exploring design trade-offs or finding inputs likely to achieve a target output. | Pareto candidates, 2D/3D surfaces, selected recipes, goal-seek recipe, CSV surface and printable HTML report. |

## 7.1 Controls

| Control | Meaning |
| --- | --- |
| Output | Dataset output to maximise, minimise or target; its unit is inherited from that column. |
| Input 1 / Input 2 | Variables shown on surface X and Y axes; their units are inherited from the dataset headers. |
| Maximise / Minimise | Direction applied to the selected output. |
| Run Pareto Optimisation | Runs one search and stores candidate recipes. |
| Target output | Desired value for inverse design, in the selected output's unit. |
| Tolerance | Maximum acceptable absolute target error, in the same output unit. |
| Run Goal Seek | Searches all inputs for a recipe near the target. |

## 7.2 Procedure

1. Confirm the active dataset and verify input/output units from its headers.

2. Select the output, two different input axes and Maximise or Minimise.

3. Select Run Pareto Optimisation and wait for candidate recipes.

4. Review model train/test R², fragmentation target band and candidate count.

5. Click or hover the surface to inspect the complete recipe and all predicted outputs.

6. Change the display axes to reproject the saved run; this does not rerun the optimiser.

7. For inverse design, enter target and tolerance and select Run Goal Seek (All Inputs).

8. Export the surface CSV or optimisation report and obtain engineering review.

UNIT RULE This module does not impose one fixed unit dictionary. It inherits names and units from the active dataset. Include units in dataset headers and never combine columns with inconsistent units.

Figure 6 — Parameter Optimisation module and run controls.

# 8. Cost Optimisation

| Purpose | Use this module when | Primary outputs |
| --- | --- | --- |
| Compute blast cost and engineering KPIs, optimise geometry under constraints and explore non-dominated cost/quality/safety trade-offs. | Balancing budget, fragmentation, vibration and airblast for candidate designs. | Engineering report, KPI cards, cost/penalty charts, constraint checks, Pareto CSVs and printable HTML report. |

## 8.1 Input groups and defaults

| Input | Default | Unit / purpose |
| --- | --- | --- |
| Diameter | 102 | mm |
| Bench height / Burden / Spacing / Subdrilling / Stemming | 10 / 3 / 3.3 / 2 / 1.8 | m |
| Number of holes / HPD | 30 / 1 | counts |
| Block volume | 0 | m³; 0 calculates B × S × bench × holes |
| Explosive density | 1.15 | g/cc |
| RWS | 115 | relative strength |
| Initiation / explosive / drilling cost | 10 / 4 / 7 | BWP per hole / per kg / per m |
| Distance R | 500 | m |
| PPV K / β / limit | 1000 / 1.6 / 12.5 | site constants; limit in mm/s |
| Airblast K_air / B_air / limit | 170 / 20 / 134 | site constants; limit in dB |
| Rock factor A / RR n | 22 / 1.8 | fragmentation constants |
| Target X50 / oversize threshold / allowed oversize | 120 / 500 / 10 | mm / mm / % |
| Burden bounds | 2.5–4.5 | m |
| Spacing:B, stemming:B, subdrill:B, stiffness bounds | Interface defaults | dimensionless engineering constraints |

## 8.2 Actions and outputs

1. Enter geometry, explosive properties, costs, site constants, limits and engineering constraints.

2. Select objective mode: Min Cost; Min Cost + Frag; or Min Cost + Frag + PPV/Air.

3. Confirm weights and objective toggles.

4. Select Compute KPIs to evaluate the current design.

5. Review total cost in BWP, PPV in mm/s, airblast in dB, X50 in mm, oversize %, PF in kg/m³ and every constraint status.

6. For a single constrained solution, use SLSQP and select Optimise.

7. For trade-off exploration, choose Pareto and select Optimise; inspect multiple frontier rows.

8. Export all/frontier rows as CSV or export the cost report.

COST ASSUMPTION The interface labels cost in Botswana pula (BWP). Confirm that all entered rates use the same currency basis, tax basis and effective date before comparing scenarios.

Figure 7 — Cost Optimisation input groups and controls.

Figure 7A — Cost Optimisation settings and engineering report.

# 9. Delay Design & Simulation

| Purpose | Use this module when | Primary outputs |
| --- | --- | --- |
| Import hole coordinates, create transparent delay assignments, validate the design, simulate firing order and export planning drafts. | Planning and reviewing tie-up timing without communicating with initiation hardware. | Interactive plan, simulation, validation, delay-assignment CSV, project JSON and printable HTML report. |

PLANNING DRAFT ONLY Do not use this module to arm, fire or directly control explosives. Do not connect it to detonator hardware. Every exported timing design requires qualified review and approval.

## 9.1 CSV input

| Column | Required | Unit / behaviour |
| --- | --- | --- |
| X | Yes | Coordinate/easting; normally m, confirm site system |
| Y | Yes | Coordinate/northing; normally m, confirm site system |
| Hole ID | No | Text; blank/N/A values receive H001-style temporary IDs |
| Z | No | Elevation/project coordinate unit; normally m |
| Depth | No | m recommended |
| Charge | No | kg recommended |

Example:

Hole ID,Depth,Charge,X,Y,Z
N1,14.454,616.027,-5363.884,4603.971,678.454

## 9.2 Timing controls

| Control | Default | Meaning |
| --- | --- | --- |
| Pattern | Row-by-row | Manual, row-by-row, chevron, V-cut, box-cut/centre-out, point-directional or line-directional |
| Start delay | 0 ms | Delay assigned to the first initiation point/group |
| In-row delay | 17 ms | Increment between ordered holes in a row/group |
| Row-to-row delay | 42 ms | Increment between detected rows/rings |
| Row tolerance | Auto; initial 8 | Coordinate bin width used to detect rows; unit follows coordinates |
| Min / max delay | 0 / 10000 ms | Allowed validation range |
| Rounding increment | 1 ms | Increment used when delay rounding is enabled |
| Direction | Left to right | Coordinate ordering direction |
| Reverse firing order | Off | Reverses computed order |
| Apply rounding | On | Rounds delay to nearest configured increment |

## 9.3 Timing patterns

| Pattern | Purpose and selection |
| --- | --- |
| Manual | Select holes in the intended firing sequence, then assign delays. |
| Row-by-row | Detected rows fire in order; holes use column order. |
| Chevron | Select a centre; holes progress centre-out within detected rows. |
| V-cut | Select an apex; holes progress by distance and V-shape spreading. |
| Box-cut / centre-out | Select a centre; inner distance rings fire before outer rings. |
| Directional from point | Select an initiation point; nearer holes fire first. |
| Directional from line | Select two holes; the last two define the initiation line. |

## 9.4 Procedure

1. Enter a project name and import a CSV, or use Load sample CSV.

2. Confirm automatic mapping for Hole ID, X, Y, Z, Depth and Charge; apply corrected mapping if required.

3. Review Validation. Fix all X/Y errors and investigate missing IDs, duplicates, missing depth/charge and outliers.

4. Choose a timing pattern. Select the required centre, apex, order or two-point line on the plan.

5. Enter timing settings and select Assign delays.

6. Review the equal-aspect plan, firing order, timing groups and analysis indicators.

7. Use Play, Pause, Reset, Step next, timeline and 0.25×–5× speed to inspect the sequence.

8. Select holes to inspect or manually edit delay values.

9. Use colour modes for delay, row, charge, depth, timing group, fragmentation, PPV, airblast or flyrock screening.

10. Resolve validation errors, then export the Planning/Simulation Draft CSV, project JSON or printable report.

## 9.5 Output formats

| Output | Filename / contents |
| --- | --- |
| Delay CSV | <project>_delay_assignment_planning_draft.csv; IDs, coordinates, timing, estimates, warnings and draft note |
| Project JSON | <project>_project.json; project settings, holes, timings and timestamps |
| Printable report | New browser tab or <project>_delay_report_planning_draft.html if pop-up is blocked |

## 9.6 Screening indicators

The module displays completeness, holes on the same delay, charge summaries, fragmentation X50, PPV, airblast and flyrock screening. These are simplified, uncalibrated planning estimates. They do not guarantee field performance or safety.

Figure 8 — Delay Design & Simulation workspace.

# 10. Slope Stability

| Purpose | Use this module when | Primary outputs |
| --- | --- | --- |
| Screen a slope condition as likely Stable or Failure using a model or local factor-of-safety-style fallback. | A rapid preliminary geotechnical screening that will be escalated for engineering assessment. | Stable/Failure classification, stable probability, train/test accuracy, class balance and slope sketch. |

## 10.1 Inputs

| Input | Range in interface | Unit / meaning |
| --- | --- | --- |
| H | 1–50 | m; slope height |
| β | 5–80 | degrees; slope angle |
| c | 1–200 | kPa; cohesion |
| φ | 5–60 | degrees; friction angle |
| γ | 14–28 | kN/m³; unit weight |
| ru | 0–1 | dimensionless pore-pressure ratio |
| B | 0–30 | m; sketch width only, not a model input |

## 10.2 File format and procedure

The upload accepts CSV, XLSX or XLS. Required model fields correspond to γ, c, φ, β, H, ru and Status. Status should identify Stable or Failure. Use clear ASCII headers where possible, for example gamma_kN_m3,c_kPa,phi_deg,beta_deg,H_m,ru,status.

1. Load an approved labelled slope dataset, or use the configured default.

2. Select Load & Predict to prepare the model and initialise parameters.

3. Adjust H, β, c, φ, γ and ru for the case. B changes only the sketch.

4. Review Stable/Failure and probability, plus train/test accuracy and class balance.

5. Escalate uncertain, out-of-range or failure-prone cases for a full geotechnical assessment.

FALLBACK BEHAVIOUR If the backend cannot provide a model response, the interface can silently display a local factor-of-safety-style estimate. Confirm model provenance before treating the probability as an ML result.

Figure 9 — Slope Stability inputs and slope sketch.

# 11. Back Break

| Purpose | Use this module when | Primary outputs |
| --- | --- | --- |
| Train a Random Forest on historical wall-control data and estimate back-break distance for an input scenario. | Comparing perimeter-control scenarios and identifying variables associated with wall damage. | Predicted back break, train/test R², feature importance and response surface. |

## 11.1 CSV format

| Typical column | Unit / role |
| --- | --- |
| Burden | m; model feature |
| Spacing | m; model feature |
| Stemming | m; model feature |
| Powder Factor | kg/m³; model feature |
| Stiffness ratio | dimensionless; model feature |
| Backbreak | m; required/automatically inferred target |

## 11.2 Procedure

1. Select a representative historical CSV with numeric features and a back-break target.

2. Select Predict Now.

3. Review predicted back break in metres and train/test R².

4. Adjust feature sliders within observed ranges; use Reset to Medians when needed.

5. Review feature importance and choose two different surface axes.

6. Use the result with Slope Stability and Flyrock screening; do not extrapolate beyond credible site ranges.

Figure 10 — Back Break data and prediction controls.

# 12. Flyrock (ML + Empirical)

| Purpose | Use this module when | Primary outputs |
| --- | --- | --- |
| Estimate flyrock throw distance with a trained model and compare it with an automatically selected empirical method. | Screening throw risk and informing a separately approved exclusion-zone assessment. | Predicted distance, empirical distance, train/test R², feature importance and response surface. |

## 12.1 CSV format

| Typical column | Unit / role |
| --- | --- |
| Hole diameter | mm; feature |
| Burden / Spacing / Stemming / Bench height | m; features |
| Charge per delay | kg; feature |
| Powder factor | kg/m³; feature |
| Rock density | t/m³; feature |
| SDoB | dimensionless scaled depth of burial; optional empirical feature |
| Flyrock distance / Throw | m; target |

## 12.2 Procedure

1. Load a representative CSV containing a recognised flyrock/throw target.

2. Select Predict.

3. Review the ML prediction and the empirical estimate in metres, including the selected empirical method.

4. Check train/test R² and feature importance.

5. Adjust input sliders only within supported data ranges.

6. Select two different surface axes or Redraw surface to inspect sensitivity.

7. Compare the output with the approved site limit and exclusion-zone method; apply the more conservative qualified assessment.

EXCLUSION-ZONE WARNING The flyrock estimate is not an automatic clearance distance. Exclusion zones must be established by qualified personnel using legislation, mine procedures, blast-specific hazards, empirical evidence and conservative controls.

Figure 11 — Flyrock ML and empirical analysis controls.

# 13. File formats and output handling

| Module | Accepted input | Exports / visible outputs |
| --- | --- | --- |
| Data Manager | CSV | CSV; nominal XLSX; site-model JSON; plots |
| Prediction | Active dataset; uploaded CSV through Data Manager | prediction_result.csv; printable HTML report |
| Feature Importance | Active dataset; uploaded CSV through Data Manager | On-screen diagnostics and plots |
| Parameter Optimisation | Active dataset; uploaded CSV through Data Manager | param_surface.csv; printable HTML report |
| Cost Optimisation | Manual numeric inputs | Cost report HTML; Pareto all/frontier CSV |
| Delay Design & Simulation | CSV | Planning-draft CSV; project JSON; printable HTML report |
| Slope Stability | CSV, XLSX, XLS | On-screen classification and sketch |
| Back Break | CSV | On-screen prediction, importance and surface |
| Flyrock | CSV | On-screen ML/empirical prediction, importance and surface |

## 13.1 CSV rules

Use one header row and one observation or hole per subsequent row.

Use a period as the decimal separator and do not include thousands separators in numeric cells.

Put units in headers, not in numeric values.

Preserve exact column names expected by the module or verify automatic mapping.

Remove merged cells, formulas returning errors, embedded notes and hidden totals before import.

Retain the original source file and create a working copy for cleaning.

## 13.2 Printable HTML reports

Prediction, Parameter Optimisation, Cost Optimisation and Delay Design generate printable HTML. The report opens in a browser tab; use the browser Print command to print or Save as PDF. If a pop-up is blocked, allow pop-ups for the approved application or use the downloaded HTML file.

## 13.3 Record retention

Record source dataset filename, revision/date, selected combined dataset and any filters.

Record all site constants, limits, optimisation weights and manual overrides.

Retain exported results with qualified review, approval and operational records under the site's document-control policy.

Do not include authentication tokens, one-time codes or unnecessary personal data in exports.

# 14. Troubleshooting

| Symptom | Likely cause | Action |
| --- | --- | --- |
| Email not authorised | Address is not in frontend/backend allowlists or deployed lists differ. | Confirm spelling and ask the access owner to verify both deployed allowlists. |
| No verification code | Mail delay, spam filtering or InstantDB configuration. | Wait briefly, check junk/quarantine, retry once and contact support without sharing codes. |
| Missing InstantDB app ID | Authentication environment variable is absent. | Administrator must configure VITE_INSTANTDB_APP_ID. |
| Backend offline/error | API URL, service, CORS, authentication or network issue. | Check status indicator and approved service status; provide timestamp and module to support. |
| CSV loads incorrectly | Wrong delimiter, quoted fields, blank headers or unit text. | Save as UTF-8 comma-separated CSV with one header row. |
| Prediction inputs remain loading | Metadata endpoint or dataset is unavailable. | Confirm backend and combined dataset; refresh after service recovery. |
| No ML output | Model assets unavailable or dataset cannot train a model. | Use empirical output cautiously and ask the model owner to check assets/data. |
| Poor/negative test R² | Weak, small or non-representative dataset. | Do not rely on the model; improve data and obtain technical review. |
| Parameter optimisation times out | Search workload or backend response time. | Retry once, use valid distinct axes and review dataset size/ranges. |
| Constraint shows Check | Current or optimised design violates a configured limit. | Review the specific ratio/limit; do not approve until resolved by qualified personnel. |
| Delay CSV has no holes | X/Y columns missing or not mapped. | Map valid numeric X and Y columns and re-import. |
| Printable report does not open | Browser blocked the new tab. | Allow pop-ups for the approved site or use the downloaded HTML fallback. |
| Excel export will not open correctly | Current Data Manager export is CSV content with .xlsx name. | Use Export Filtered → CSV; convert to XLSX in spreadsheet software if required. |

## 14.1 Information to provide to support

Date/time, module, browser and whether the backend status appeared online.

Dataset filename and column headers only, unless the support channel is approved for operational data.

The exact visible error message and steps immediately before it.

A redacted screenshot with secrets, codes and sensitive production values removed.

Never send passwords, one-time codes, refresh tokens or API keys.

OWNER ACTION REQUIRED — SUPPORT DETAILS Insert the approved service desk email/phone, escalation path, operating hours, incident severity rules and production status-page address.

# 15. Glossary

| Term | Definition |
| --- | --- |
| Airblast | Pressure wave transmitted through air and represented here as a dB level. |
| Back break | Rock breakage extending behind the intended final wall or last row. |
| Empirical model | Equation fitted or calibrated from observed practice rather than a learned black-box model. |
| Flyrock | Rock projected beyond the intended blast area. |
| Kuz–Ram | Empirical fragmentation model used to estimate a characteristic mean size. |
| Machine learning (ML) | Model trained from historical input/output observations. |
| Pareto frontier | Solutions for which one objective cannot improve without worsening another. |
| PCA | Principal component analysis; transforms correlated variables into components explaining variance. |
| Permutation importance | Model performance loss when one feature is shuffled. |
| Rosin–Rammler (RR) | Particle-size distribution model using characteristic size and uniformity n. |
| Scaled burden | Burden normalised by a charge-related term for screening confinement/throw risk. |
| Scaled distance | Distance normalised by charge per delay for vibration/airblast relationships. |
| Surrogate model | Faster approximation used to explore or optimise an expensive response. |

# 16. Owner completion checklist

Complete and approve every item below before issuing this manual as a controlled production document.

| Item | Required owner action | Complete |
| --- | --- | --- |
| Production identity | Add organisation logo/name, production URL, software build/release and document identifier. | ☐ |
| Approval | Add document owner, technical reviewer, qualified blasting reviewer and release signatures. | ☐ |
| Access list | Verify deployed frontend/backend allowlists and define access review/revocation process. | ☐ |
| Support | Add service desk and escalation details. | ☐ |
| Site constants | Document approved PPV, airblast, fragmentation and flyrock limits and calibrated constants. | ☐ |
| Coordinate system | State the approved X/Y/Z coordinate reference system and units. | ☐ |
| Cost basis | Confirm currency, effective date, tax basis and cost-rate ownership. | ☐ |
| Model governance | Add model/dataset owners, approved datasets, validation status and retraining controls. | ☐ |
| Screenshots | Replace any yellow screenshot placeholders with approved production screenshots. | ☐ |
| Excel limitation | Correct or formally document the nominal XLSX export behaviour. | ☐ |
| Training | Add required competence, induction and refresher requirements. | ☐ |
| Retention | Add the applicable document and blast-record retention period. | ☐ |

FINAL RELEASE GATE A qualified blasting reviewer must confirm that the manual's limits, terminology, units, screenshots and workflows match the deployed software and local procedures before operational use.